Output torque:

T0= 25 [Nm]

Rotational speed:

wo=π20.5=3.14159 [rads]

no= 30 [rpm]

Power:

P = Towo=78.54 W

Motor and Gearbox Selection:

Key load parameters:

| Output Torque [Nm] | Rotational Speed [rpm] | Power [W] |
| --- | --- | --- |
| 25 | 30 | 78.54 |

We firstly select the gear box that can allow 25 [Nm] output torque from company Maxon. From the product catalogue [See Appendix], we select the gearbox Planetary Gearhead GP 52 C.

Free body diagram:

Force analysis:

F=mgγ=2.5 kg× 9.81 ms2×3=73.575 N

where γ is the magnification factor.

Force Diagram:

Point D is the critical point since the bending moment at this point is highest and there is a stress concentration of the guide rail.

The internal moment and the shear force at the critical point:

V = F

Mz=V*A=110.363 Nm

Let’s choose G10300 HR steel as the material from Shigley’s book Table A-20, the properties:

Dimension of the beam:

Thickness: h = 0.015 m

Width: b = 0.015 m

Let’s check for the yielding:

Choose the safety factor as

n = 1.3

Bending stress:

Normal: τx=-MzyIz= -196.200000 [MPa]

Shear: τxz=VQIb=0.490500 [MPa]

Since it’s a ductile material, choose Distortion-energy criterion:

σ'=(τx2+3τxz2)12 = 196.200613 [MPa]

σ'≤Synd=2601.3= 200 [MPa]

Thus, the beam will not yield.

Appendix: